Steady-state aerodynamics of a Porsche Taycan: A numerical investigation at Re=7×106

Name:                LEI, Zhen

Student ID:            20937857

Course:       Computational Fluid Dynamics

Instructor:            Prof. Larry Li

Department of Mechanical and Aerospace Engineering,

The Hong Kong University of Science and Technology

05/2023

Content

1. Introduction and engineering problem	3

1.1 Background information	3

1.2 Case application	3

2. Numerical method	3

2.1 Case assumptions	3

2.2 Governing equations	3

2.3 Numerical approach	4

3. Numerical setup	4

3.1 Computational domain	4

3.2 Boundary/Initial conditions	5

4. Grid convergence test	6

4.1 Test matrix	6

4.2 Results and discussion.	7

5. Conclusions	7

Reference	7

1. Introduction and engineering problem

In this case, there is steady-state aerodynamics of a Porsche Taycan: A numerical investigation at Re=5×106. The description of the engineering problem is as below:

- The car Porsche Taycan moving at a speed of 20m/s.

- Air flows around the body of the car.

- Assume sea-level air properties at 15°C.

The schematic of the engineering problem could be seen as Figure 1:

Figure 1: schematic of the engineering problem

1.1 Background information

1.2 Case application

2. Numerical method

2.1 Case assumptions

The flow of the CFD case satisfies the conservation of mass and conservation of momentum.

Also, there are some basic assumptions which is used to simplify the engineer problem.

- The flow is incompressible, and it is constant.

- The flow is steady, which means  ∂∂t=0.

2.2 Governing equations

As the case is a 3D case, ∂/∂z should be taken into consideration. As a result, the governing equations for the case are:

Mass: #2.1

Momentum: #2.2

2.3 Numerical approach

Just as mentioned, this case is the numerical simulation of the steady flow around a 3D car, so steady solvers are in need. Based on the governing equations, simpleFoam is selected for simulating this case in unsteady flow.

SimpleFoam is a steady-state solver for incompressible, turbulent flows using the SIMPLE algorithm. SIMPLE algorithms are based on evaluating some initial solutions and then correcting them. SIMPLE only makes 1 correction. The grid convergence test will be conducted for different mesh to get the result.

3. Numerical setup

The flow properties used in this case are as Table 1.

Table 1: flow properties

| Parameters | Value |
| --- | --- |
| Reynolds Number (Re=ρUDμ) | 6.85×106 |
| Length (L) | 5m |
| Air density (ρ) | 1.225kg/m3 |
| Kinematic Viscosity (ν=μρ) | 1.46*10-5m2/s |
| Velocity (U) | 20m/s |

3.1 Computational domain

The computational domain could be as Figure 2.

Figure 2: 3D case for the computational domain

Figure 3: breakdown of the computational domain

3.2 Boundary/Initial conditions

Here are summaries of the boundary conditions and initial conditions:

| Turbulence Model |  | k-ω SST |  |
| --- | --- | --- | --- |
| y+ |  | ≈100 |  |
| BC | Boundary | Type | Value |
|  | inlet | U,k,omega:fixedValue p:zeroGradient nut:calculated boundary:patch | U= (20 0 0) nut=0 k=0.06 omega=0.089 |
|  | outlet | U,k,omega: inletOutlet p: fixedValue nut:calculated boundary:patch | U: inletValue 0, value (20 0 0) p=0 nut=0 k: inletValue 0.06, value 0.06 omega: inletValue 0.089, value 0.089 |
| BC (continued) | LowerWall | U: fixedValue p:zeroGradient nut: nutkWallFunction k: kqRWallFunction omega: omegaWallFunction boundary:wall | U= (20 0 0) p=0 nut=0 k=0.06 omega=0.089 |
|  | walls | U: Noslip p: zeroGradient nut: nutkWallFunction k: kqRWallFunction omega: omegaWallFunction | nut=0 k=0.06 omega=0.089 |
|  | upperWall | U,P,k,omega:slip nut:calculated boundary:patch | Nut=0 |
|  | frontAndBack | U,P,k,omega: slip nut:calculated boundary:patch | Nut=0 |
| IC | sides | - |  |
|  | inlet | U = (20,0,0) |  |
|  | topBot | - |  |
|  | outlet | p = 0 |  |
|  | wall | - |  |

4. Grid convergence test

4.1 Test matrix

In this part, we are going to conduct the grid convergence test of the CFD case, set different refinement levels as Table 2.

Table 2: test matrix

| mesh | Level 0 Edge Length (m) | cells | y+ |
| --- | --- | --- | --- |
| Coarse | 0.91 | 265391 | ≈270 |
| Medium 1 | 0.70 | 549497 | ≈170 |
| Medium 2 | 0.53 | 1204214 | ≈110 |
| Fine | 0.48 | 1459630 | ≈100 |

Figure 4: grid test

4.2 Results and discussion.

Obtain cd from the equation

D=12cdρV2S#4.1

Where ρ is the air density, V is the freestream velocity, S is the front area of taycan, the value is 2.33 m2.

Figure 5 is the drag coefficicent for different mesh.

Figure 5: cd for different mesh

Calculate the mean value for cd in every 100 iterations, obtain the trend for cd in Figure 6.

Figure 6: trend for cd

Obtain the cd=0.22 for porsche taycan from https://media.porsche.com/mediakit/taycan/en/porsche-taycan/die-aerodynamik as ground truth.

Table 3: Result for grid test

| mesh | cells | cd | Error for cd (%) |  | Time (s) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coarse | 265391 | 0.386 | 75 | 546 |  |
| Medium 1 | 549497 | 0.387 | 76 | 1149 |  |
| Medium 2 | 1204214 | 0.379 | 72 | 2858 |  |
| Fine | 1459630 | 0.376 | 71 | 6635 |  |

Figure 7: error and time analyze

It could be concluded vividly that the result from numerical method is sharply different from the experimental data reported by the official resource. There are several assumptions for the error:

- The official resource always report their best data for aerodynamic performance which may could only be conducted in specific condition, this time is not in except.

- Turbulence model doesn’t fit this case, maybe Spalart-Allmaras or k-ε could have a better performance in this case.

- The mesh is not good enough, especially the Boundary layer, and the y+ is not lower enough.

- Before conducting the numerical part, the stl model of the car has been modified in order to simplyfy the case, for example, the wheel has been replaced by cylinder. This may affect the aerodynamic performance.

- There is an original ∆cd for the CFD solver, which indicates that the numerical method has its drawback.

Meanwhile, the results shows with the increase of the cells, the accuracy could have a rise while the computational time will be strictly increase, which is in need of striking a balance.

5. Conclusions

Reference